# Project 03: Gamepad Driving

In this project you drive the robot with the two thumbsticks. One stick runs the left wheel. The other runs the right wheel.

Save your own copy first. In Thonny, open p03_gamepad_driving.py and choose File > Save As. Pick the Alvik (the MicroPython device) and save it as /workspace/p03.py. Do all your work in that copy. Files outside /workspace get overwritten when the projects are updated.

# Part 1: How it works

Read this part when you get stuck. It has the answer to almost every question you will have.

## What is already in your file

Your file comes with the setup lines, the loop, and the shutdown line. The three # WORK blocks are empty.

You fill them by typing the code out of Part 2 by hand, the same way you did in P01 and P02. Thonny still does the indenting.

## How tank drive steers

A car steers with a steering wheel. A tank does not. A tank has a motor on each side, and it turns by running one side faster than the other.

Your robot drives the same way. That is why this is called tank drive.

- Both sticks forward, and it goes straight.

- Push the left stick further than the right, and it curves right.

- Left stick forward, right stick back, and it spins in place.

Nothing in your code will say the word "turn." A turn is just two wheels doing different things.

## What the sticks tell you

Your file already has the loop and the refresh:

while not (sb.held('cancel') or gamepad.held('options')): gamepad.update()

gamepad.update() asks the controller what it is doing right now. Leave it out and you read the same stale numbers forever.

After the update, two values are ready for you:

gamepad.left_y # the left stick gamepad.right_y # the right stick

Each one is a decimal between -1.0 and +1.0.

- All the way forward: +1.0

- Let go, centered: 0.0

- All the way back: -1.0

Halfway forward is about +0.5. The stick reports how far you pushed it, not just which way. That is what makes smooth driving possible.

## SuperBot can print for you

You cannot see inside a running robot. There is no way to know what gamepad.left_y is right now by staring at the robot. The suit has a power for that:

sb.log_info(gamepad.left_y)

Whatever you hand it gets printed in the Thonny Shell and shown on the robot's little screen at the same time. Push the left stick about halfway forward and that line prints:

0.51

Hand it two values and you get both, side by side:

sb.log_info(gamepad.left_y, gamepad.right_y)

0.51 -0.23

That is the third power you have used, after sb.held and sb.light_both_leds.

## One press, one reading

Now try to use it. Drop sb.log_info into the loop on its own:

sb.log_info(gamepad.left_y, gamepad.right_y)

Every trip through the loop prints a line, and the loop runs about fifty times a second:

0.0 0.0 0.0 0.0 0.0 0.0 ... fifty more of these, every second

The numbers scroll past faster than you can read them.

So put it behind a button. You already used held up in the loop line, and it looks like the obvious tool:

if gamepad.held('cross'): sb.log_info(gamepad.left_y, gamepad.right_y)

Better, but not much. held is True the whole time the button is down, and nobody can tap a button for less than a fiftieth of a second. You still get a burst of ten or twenty lines. You just get them in clumps now.

What you want is one line per press:

gamepad.pressed('cross')

pressed is True only at the instant the button goes down. Hold X as long as you like and you get one True. Let go and press again for a second reading.

Both words work on both devices. sb.held('ok') and sb.pressed('ok') ask the same two questions of the robot's own pads.

## Turning stick numbers into wheel speeds

Say you push the left stick halfway, get +0.5, and send that straight to the wheels. The robot barely twitches.

Nothing is broken. The motors do not measure speed the way sticks do. Motors want RPM, which is rotations per minute, and you just asked for half a rotation a minute.

So you translate. You multiply:

target_rpm = stick_value * MAX_RPM

MAX_RPM is already set to 70 in your file. That is as fast as these motors go. Push the stick halfway to +0.5 and you get 35 RPM. Push it all the way to +1.0 and you get the full 70.

Here is the nice part. The minus sign takes care of itself. Pull back to -1.0 and the math gives you -70, and a negative RPM means the wheel turns backward. You never write any code for reverse. It comes free.

## Sending the speeds to the wheels

One line drives both wheels:

alvik.set_wheels_speed(left_speed, right_speed)

Left first, right second.

Call it every time through the loop. The wheels keep doing whatever you told them last. Stop calling it and the robot keeps rolling.

## Cleaning up when there are motors

You met finally in P01. Code inside it always runs. Normal ending, crash, or Stop button.

It matters more now. In P01 a crash left your lights stuck on, which is annoying. In this project a crash leaves the wheels spinning, and the robot drives itself off the table.

So the first thing in your cleanup is this:

alvik.brake()

That stops both motors.

There is one more thing to put right. Back at the top, when the gamepad connected, it turned both lights green to tell you so. Nothing in this project ever changes them again, so they are still green when you quit. Clear them with the power you met in P02:

sb.light_both_leds(0, 0, 0)

The last line, alvik.stop(), is already written for you. Leave it there. Both of your cleanup lines go above it.

# Part 2: Do the work

## Step 1: Set up

- Turn on your Alvik robot.

- Open p03_gamepad_driving.py in Thonny and save your own copy to the robot, as described at the top of this guide.

- Run the program. Thonny prints your robot's WiFi name in the Shell.

- Connect your Mac to that WiFi network. The password is password.

- Open http://192.168.4.1 in Chrome. Keep that Chrome window in front for the whole project. Chrome blocks gamepad input to any window that is not focused.

- Pair your PS5 controller to the Mac over Bluetooth. The robot's LEDs turn green when it connects.

- Put the robot on the floor. Not on a table.

## Step 2: Type in WORK 1

Look before you drive. These two lines print both stick values, once, each time you press X.

Find the # WORK 1 comment and type them underneath it:

if gamepad.pressed('cross'): sb.log_info(gamepad.left_y, gamepad.right_y)

Run it. Push a stick all the way and press X. Push it halfway and press X. Let go and press X. Write all three readings down.

You should see the full push land near 1.0, the half push near 0.5, and the centered stick sit at exactly 0.0.

## Step 3: Type in WORK 2

Now the driving. Two lines of math, then one line that sends it.

Find the # WORK 2 comment and type these underneath it:

left_speed = gamepad.left_y * MAX_RPM right_speed = gamepad.right_y * MAX_RPM alvik.set_wheels_speed(left_speed, right_speed)

Run it and drive. Straight, curving, spinning in place, backward. Your WORK 1 readings still print whenever you press X.

## Step 4: Type in WORK 3

This is the shutdown, and with motors it is the part that keeps your robot on the table.

Find the # WORK 3 comment, inside finally:, and type these underneath it. They go above the alvik.stop() line that is already there:

alvik.brake() sb.light_both_leds(0, 0, 0)

Test it by pressing Options while the robot is rolling. The wheels should stop right where they are.

## Step 5: Worksheet and check off

One run shows all of it. Press X a few times and read your numbers out of the Shell, then drive forward, back up, spin in place, and press Options to stop clean.

Part A of the worksheet has three WORK boxes and a FLEX box. Those match the # WORK and # FLEX comments in your Python file. Copy the key line or lines from each one into its box. If you skipped the flex, leave the FLEX box empty.

One checkoff. Run the finished file once and it shows everything you built, so nothing gets commented out along the way. The work is 19 points, the flex is 20, undone is 0. Late is your points × 0.9. No robot or no worksheet is a 0 and a redo. Work at your own speed and watch the due dates. Finish all fourteen and the Lego and servo bin unlocks.

## FLEX: The A+

This one is not printed anywhere. You work it out.

Make each LED report on its own wheel.

- That wheel going forward: the LED on that side is green.

- That wheel going backward: red.

- That wheel stopped: white.

sb.light_both_leds will not help you here. It sets both lights the same, and the whole point is that the two sides can disagree.